GETTING SET UP:

- Move the EVLENGTH and FREEFALL files to the microSD card you want to use (located in /Settings Files)

- Install the Sparkfun AVR pocket Programmer Drivers:

  - Follow: https://learn.sparkfun.com/tutorials/pocket-avr-programmer-hookup-guide

- Install the FTDI drivers:

  - Follow: https://learn.sparkfun.com/tutorials/how-to-install-ftdi-drivers

PROGRAMMING NEW DEVICES:

- Install the Arduino IDE (located in /Arduino Installers)

- In the Arduino IDE:

  - Tools -> Board, and select Arduino Pro or Pro Mini

  - Tools -> Processor, select ATmega328 (3.3V, 8MHz)

- Open the Capstone_46_SetRTC program (located in Firmware/Capstone_46_SetRTC/)

- Connect the Sparkfun FTDI Basic Breakout to your computer USB

- Connect the Sparkfun Pocket AVR Programmer to your computer USB

- Connect the Sparkfun FTDI Basic Breakout to the device JP1 (*see image)

- Connect the Sparkfun Pocket AVR Programmer to the device 2x3 header below the RST button (*see image) (MAKE SURE IT IS SET TO “No Power”)

  - (both the fdti and the AVR programmer should be connected)

- Tools -> Programmer, select USBtinyISP

- Make sure the Power switch is UP!!! (towards the PWR label)

- Tools -> Burn Bootloader

- Wait until the Arduino IDE indicates that the bootloader was burned successfully

- Remove the 2x3 AVR programmer connector and unplug the USB

- Flip the power switch Down

- File -> Upload

- Wait until the Arduino IDE indicates the program (the RTC program in this case) was uploaded properly

- Tools -> Serial Monitor

- Follow the prompts to set the time on the RTC

- Close the serial window

- Insert the microSD card into the device

- Now open the Capstone_46_Proto2_SLOWSD program (located in Firmware/Capstone_46_Proto2_SLOWSD)

- File -> Upload

- Remove the FTDI programmer, and flick the power switch up to turn device off!

USE

- Make sure microSD has EVLENGTH and FREEFALL files

  - EVLENGTH = 1 to 9999 (500-2000 recommended)

  - FREEFALL = 1 to 60 (10-20 recommended)

- Put microSD into device

- Turn device on (down = on, up = off)

  - Green LED should light

- Press record (REC) to start

  - Green LED should turn off

- Close device and put on patient

- Get device back from patient

- Open device

  - Be careful not to drop to trigger recording (bug when stopping during recording)

- Press record again to stop

  - Green LED should flash indicating completion

    - *if not, data may still be ok! Just keep going

- Turn device off (up = off) & remove microSD card, and insert into computer

- Open Fall Data Analyser (/Excel Macro/)

- Click on button and select data off of microSD card